# Appendix

# Appendix A

## Tank Geometry Relations

Covers the four tank shapes the class supports: spherical, cylindrical with hemispherical domes, cylindrical with 2:1 semi-elliptical (ellipsoidal) domes, and fully ellipsoidal.

Table 1. Nomenclature

| d | Tank diameter |
| --- | --- |
| r=d/2 | Tank radius |
| H | Overall tank length (pole to pole) |
| hd | Dome/head height (depth of one end cap) |
| hb (hb=H-2hd) | Cylindrical barrel length |
| Vand A | Total tank volume and surface area. |
| Vl and Vg | Instantaneous liquid volume and ullage (vapor) volume. |
| h | Liquid height from the tank’s lowest point. |
| w | Free-surface (liquid-vapor interface) width. |
| Aint and Lint | Interface area and characteristic length. |
| SAwet and SAdry | Wetted and dry internal surface area |

## A.1 Spherical tank

r=d/2, hd=r

V=43πr3  SA=4πr2

Liquid height, 0≤h≤2r:

Vl=πrh2-π3h3

Free-surface width:

w=2h2r-h

Wetted area:

Awet=2πrh,  SAdry=SA-SAwet

## A.2 Cylindrical tank with hemispherical domes

r=d/2, hd=r, hb=H-2r

V=πr2hb+43πr3  SA=2πrhb+4πr2

Vcap=23πr3,  Vbarrel=πr2hb

Region 1, bottom dome (0≤Vl≤Vcap):

Vl=πrh2-π3h3,  w=2h2r-h

Region 2, barrel (Vcap<Vl≤Vcap+Vbarrel):

h=Vl-Vcapπr2+r,  w=2r

Region 3, top dome (Vl>Vcap+Vbarrel), with Vc=Vl-Vbarrel and auxiliary height y:

Vc=πry2-π3y3,  h=y+hb,  w=2y2r-y

Wetted area:

Awet=2πrh,h≤r2πr2+2πrh-r,r<h≤r+hb2πrh-hb+2πrhb,h>r+hb

SAdry=SA-SAwet

## A.3 Cylindrical tank with 2:1 semi-elliptical domes

r=d/2, hd=d/4, hb=H-2hd

Dome semi-axes: a=b=r, c=hd.

V=πr2hb+43πr2hd

A=2πrhb+4πr⋅r1.6+r⋅hd1.6+r⋅hd1.631/1.6

Vcap=23πhd3,  Vbarrel=πr2hb

Region 1, bottom dome (0≤Vl≤Vcap):

Vl=πd316hd2h2-πd348hd3h3,  w=2r

Region 2, barrel:

h=Vl-Vcapπr2+hd,  w=2r

Region 3, top dome, with Vf=V-Vl and auxiliary height y:

Vf=πd316hd2y2-πd348hd3y3,  h=2hd+hb-y,  w=2r

Shorthand:

Re=r2-hd2

Region 1 (h≤hd):

SAwet=πr2-πrhd2hd-hhd4+hd-h2Re2+πhd2rRelnhdRe+rhd-hRe+hd4+hd-h2Re2

Region 2 (hd<h≤hd+hb):

Awet=πr2+πhd2r2Relnr+Rer-Re+2πrh-hd

Region 3 (h>hd+hb), with f=hb+2hd-h:

Adry,cap=πr2-πrhd2fhd4+f2Re2+πhd2rRelnhdRe+rfRe+hd4+f2Re2

Awet=2πr2-SAdry,cap,  SAdry=SA-SAwet

## A.4 Ellipsoidal (spheroidal) tank

a=b=d/2, c=H/2

V=43πd22H2

A=4πabp+acp+bcp31/p,  p=1.6075

With Rh=d/2 and Fz=πRh2z-z33c2:

Vcaph=Fh-c-F-c=Vl (solved numerically for h)

Free-surface width:

w=2Rh1-h-cc2

With ρz=Rh1-z/c2 and ρ'z=-Rhzc21-z/c2:

Awet=-ch-c2π ρz1+ρ'z2 dz

SAdry=SA-SAwet

## A.5 Common relations (all shapes)

Aint=πw22,  Lint=Aintπw

hullage=Vg/Aint,cylinder caseH-hint,general case

# Appendix B

## B.1 Liquid Hydrogen Fitted Fluid Properties Correlations

The saturated fluid properties were obtained from the National Institute of Standards and Technology (NIST) Chemistry WebBook [1–3] and correlated using third-degree polynomial fits. The correlations were developed over a temperature range of 18–33 K, with the critical temperature of hydrogen taken as Tc=33.145 K, and a pressure range of 0.1–0.8 MPa. Depending on the specific fluid property and the available thermophysical data, the correlations were formulated as functions of temperature Tand pressure P, while the density correlation additionally incorporated the parameter τ.

The general form of the third-degree polynomial correlation is given by

y=a0+a1x+a2x2+a3x3,

where yrepresents the corresponding saturated fluid property, xdenotes the independent variable used in the correlation (T, P, or τ, as applicable), and a0, a1, a2, and a3are the fitted polynomial coefficients. A separate polynomial correlation was developed for each saturated fluid property based on the NIST thermophysical data within the specified temperature and pressure ranges.

Coefficient table

| Liquid hydrogen saturation properties at 18-33K (Tc= 33.145 K, τ=1-TTc) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Parameter | Units | a0 | a1 | a2 | a3 | R2 | RMSE |
| ρlτ | kg/m3 | 45.289261 | 114.26525 | -175.36288 | 131.61077 | - | - |
| Psat | MPa | -0.37539036 | 0.074328435 | -0.005318205 | 0.00013836551 | - | - |
| Vl | m3/kg | -0.0040798056 | 0.0023067368 | -0.00010592399 | 1.795401e-06 | - | - |
| ul | kJ/kg | -268.11783 | 24.026933 | -0.90085605 | 0.017890514 | - | - |
| hl | kJ/kg | -310.44796 | 30.369102 | -1.2285984 | 0.023877159 | - | - |
| sl | J/g⋅K | -15.487598 | 1.3174483 | -0.040535915 | 0.0006478982 | - | - |
| cvl | J/g⋅K | 1.2252082 | 0.48075706 | -0.018320169 | 0.0002658936 | - | - |
| cpl | J/g⋅K | -137.72187 | 19.835175 | -0.90728735 | 0.01420863 | - | - |
| vsl | m/s | 1927.5301 | -79.051212 | 3.289686 | -0.066723465 | - | - |
| JTl | K/MPa | -11.489421 | 1.3182398 | -0.058297093 | 0.00093250884 | - | - |
| µl | Pa⋅s | 8.60965e-05 | -7.1309416e-06 | 2.3012752e-07 | -2.7007579e-09 | - | - |
| kl | kJ/m⋅K | 0.075561753 | 0.0016744237 | 2.787619e-05 | -2.0783363e-06 | - | - |
| Gaseous hydrogen (Vapor) saturation properties at 18-33K (Tc= 33.145 K, τ=1-TTc) |  |  |  |  |  |  |  |
| ρv(τ) | kg/m3 | 17.660638 | -93.969718 | 188.84445 | -142.03836 | - | - |
| Psat | MPa | -0.37539036 | 0.074328435 | -0.005318205 | 0.00013836551 | - | - |
| Vv | m3/kg | 28.580044 | -3.1043675 | 0.11409545 | -0.0014100093 | - | - |
| uv | kJ/kg | 423.36206 | -18.013427 | 1.2400147 | -0.023483178 | - | - |
| hv | kJ/kg | 474.18803 | -21.350443 | 1.641935 | -0.032193718 | - | - |
| sv | J/g⋅K | 66.905101 | -4.327337 | 0.14147932 | -0.0018261022 | - | - |
| cvv | J/g⋅K | 4.3458941 | 0.30053071 | -0.016346277 | 0.00032721158 | - | - |
| cpv | J/g⋅K | -187.1303 | 27.475674 | -1.2758862 | 0.020012343 | - | - |
| vsv | m/s | 104.76949 | 18.985554 | -0.32000673 | -0.00026982876 | - | - |
| JTv | K/MPa | 79.880405 | -5.9983053 | 0.19447902 | -0.0023787768 | - | - |
| μv | Pa⋅s | -2.110857e-06 | 3.4092853e-07 | -1.3731752e-08 | 2.2033704e-10 | - | - |
| kv | kJ/m⋅K | -0.077904577 | 0.01162638 | -0.00051269526 | 8.4422435e-06 | - | - |
| Liquid hydrogen saturation properties at 0.1-0.8 MPa |  |  |  |  |  |  |  |
| Tsat | K | 16.59013 | 41.262621 | -56.08262 | 32.283884 | - | - |
| Vl | m3/kg | 0.013398015 | 0.0077685385 | -0.0056689917 | 0.0045590486 | - | - |
| ul | kJ/kg | -30.910439 | 329.83255 | -303.51939 | 171.04307 | - | - |
| hl | kJ/kg | -30.982374 | 344.2248 | -300.15303 | 173.04202 | - | - |
| sl | J/g⋅K | -1.3862659 | 16.014974 | -17.776774 | 9.9275611 | - | - |
| cvl | J/g⋅K | 5.4699923 | 2.2497081 | -2.9152097 | 1.8666268 | - | - |
| cpl | J/g⋅K | 7.7808106 | 21.713057 | -23.118197 | 28.342789 | - | - |
| vsl | not stated | 1206.6239 | -968.82974 | 812.35613 | -468.42907 | - | - |
| JTl | K/MPa | -1.2670114 | 3.504617 | -3.2220159 | 2.5903203 | - | - |
| μl | Pa⋅s | 1.6266559e-05 | -3.4591383e-05 | 5.0714548e-05 | -2.9214549e-05 | - | - |
| kl | kJ/m⋅K | 0.10380407 | 0.0039625463 | -0.031392614 | 0.015666705 | - | - |
| Gaseous hydrogen saturation properties at 0.1-0.8 MPa |  |  |  |  |  |  |  |
| Tsat | K | 16.59013 | 41.262621 | -56.08262 | 32.283884 | - | - |
| Vv | m3/kg | 1.1068405 | -4.9570723 | 8.8392336 | -5.4053178 | - | - |
| uv | kJ/kg | 364.34132 | 110.7788 | -214.20356 | 97.316123 | - | - |
| hv | kJ/kg | 434.73423 | 184.61704 | -347.89395 | 163.63532 | - | - |
| sv | J/g⋅K | 24.264661 | -26.928259 | 35.308996 | -20.301053 | - | - |
| cvv | J/g⋅K | 6.3030629 | 1.4433011 | -0.16891883 | 0.26345511 | - | - |
| cpv | J/g⋅K | 10.318169 | 17.844555 | -17.630057 | 32.131225 | - | - |
| vsv | m/s | 341.81871 | 185.62638 | -307.21186 | 170.10013 | - | - |
| JTv | K/MPa | 20.959339 | -32.770819 | 45.61897 | -26.253129 | - | - |
| μv | Pa⋅s | 8.388579e-07 | 1.8099075e-06 | -1.7615907e-06 | 1.0875333e-06 | - | - |
| kv | kJ/m⋅K | 0.013519624 | 0.043672174 | -0.040572309 | 0.029790846 | - | - |

# Appendix C

## C.1 Stochl tank insulation properties

Table 2. Stochl tank, and insulation properties sourced from NASA’s Stoch et al.[4,5]  experiments

| Item | Material | Emissivity | Unit weight [kg/m²] |
| --- | --- | --- | --- |
| Radiation shield | 0.0064 mm (¼-mil) double-aluminized Mylar | <0.04 both sides | 0.00884 |
| Shield spacer | 4.7×7.1 mesh/cm² silk net, thickness ≤0.010 cm | - | ≤0.00732 (per layer) |
| Blanket cover sheet (a) (b) (c) | Schjeldahl X-850 laminate: | ≤0.05 both sides | 0.0780 |
|  | 0.0064 mm single-aluminized Mylar | ≤0.05 alum. surface | 0.00884 |
|  | Dacron scrim | - | 0.0200 |
|  | 0.013 mm single-aluminized Mylar | ≤0.05 alum. surface | 0.0177 |

# Appendix D

## D.1 MHTB Tank insulation properties

Table 3. MHTB tank, and insulation properties sourced from NASA’s Hastings et al.[6–8]  experiments

| Component | Parameter | Symbol | Value | Unit | Description / Source |
| --- | --- | --- | --- | --- | --- |
| Dacron net spacer | Spacer material | - | Dacron | - | B4A Dacron net spacer |
|  | Spacer thickness | Dx | 1.0×10-4 | m | Same construction across all regions |
|  | Spacer density | fsp | 0.0087 | - | Hastings, Table 1 |
|  | Thermal conductivity | ksp | 0.0075 | W/(m·K) | Assumption |
|  | Spacer conduction coefficient | C2 | 0.008 | - | Empirical coefficient, Hastings, Table 1 |
| Radiation shields | Shield material | - | Double-aluminized Mylar (DAM) | - | Radiation shield |
|  | Shield thickness | δins | 6.4×10-6 | m | ¼-mil Mylar |
|  | Volumetric density | ρins | 1390 | kg/m³ | Hastings, Table 1 |
|  | Specific heat capacity | cp,ins | 1000 | J/(kg·K) | Mylar |
|  | Emissivity | εins | 0.03 | - | Hastings, Table 1 |
| Residual gas | Gas species | - | N₂ | - | Residual gas |
|  | Thermal conductivity | kgas | 0.0095 | W/(m·K) | At mean MLI temperature |
|  | Molar mass | Mg | 0.028 | kg/mol | Hastings, Table 1 |
|  | Accommodation coefficient | θ | 0.8 | - | Hastings, Table 1 |
|  | Degrees of freedom | ζ | 2.0 | - | Diatomic N₂ |
|  | Specific heat ratio | γ | 1.4 | - | Hastings, Table 1 |
|  | Residual gas pressure | Pres | 1.33×10-5 | Pa | Hastings, Table 1 |
|  | Nusselt number | Nugas | 1.0 | - | Assumed |
| Outer shroud | Shroud thickness | δSe | 0.01 | m | Outer boundary |
|  | Shroud density | ρSe | 2700 | kg/m³ | - |
|  | Shroud specific heat | cp,Se | 900 | J/(kg·K) | - |
|  | Shroud emissivity | εSe | 0.04 | - | Hastings, Table 1 |
|  | Environment emissivity | εenv | 1.0 | - | LN₂ shroud approximated as blackbody |
|  | Convective heat-transfer coefficient | hconv | 0 | W/(m²·K) | No convection |
|  | Ambient temperature | T∞ | 305 | K | Warm-side boundary; varied for validation |
| Inner tank wall | Material | - | 5083 Aluminum | - | Inner tank wall |
|  | Wall thickness | δSi | 1.25×10-2 | m | NASA MHTB report |
|  | Density | ρSi | 2660 | kg/m³ | Corrected 5083-Al density |
|  | Specific heat capacity | cp,Si | 900 | J/(kg·K) | At room temperature |
| SOFI foam | Foam type | - | Isofoam / polyurethane spray-on foam | - | Spray-on foam insulation |
|  | Foam thickness | tfoam | 0.0353 | m | Hastings, Table 1 |
|  | Thermal conductivity | kfoam | 0.000866 | W/(m·K) | Hastings, Table 1 |
|  | Density | ρfoam | 32.0 | kg/m³ | Typical polyurethane foam density |
|  | Specific heat capacity | cp,foam | 1400 | J/(kg·K) | SOFI specific heat capacity |
|  | Emissivity | εfoam | 0.80 | - | Hastings, Table 1 |

# Appendix E

## E.1 VCS Properties

Table 4. Assumed input parameters for the VCS

| Component | Parameter | Symbol | Value | Unit | Description | Source |
| --- | --- | --- | --- | --- | --- | --- |
| VCS | VCS status | - | Disabled | - | Disabled for insulation-only validation | Assumption |
|  | VCS thickness | δVCS | 2.0×10-3 | m | VCS layer thickness | Assumption |
|  | Density | ρVCS | 2840 | kg/m³ | VCS material density | Assumption |
|  | Specific heat capacity | cp,VCS | 864 | J/(kg·K) | VCS material specific heat capacity | Assumption |
|  | VCS fraction | fVCS | 0.10 | - | VCS flow fraction | Assumption |
|  | Configuration | - | Series | - | VCS thermal configuration | Assumption |
|  | Partition parameter | α | 0.5 | - | VCS parallel flow fraction split | Assumption |
|  | VCS 1 location | - | LD–MD | - | Location between low-density and medium-density MLI | Assumption |
|  | VCS 2 location | - | MD–HD | - | Location between medium-density and high-density MLI | Assumption |

# Appendix F

## F.1 Venting parameters

Table 5. Assumed model venting parameters

| Component | Parameter | Symbol | Value | Unit | Description | Source |
| --- | --- | --- | --- | --- | --- | --- |
| Vent system | Vent status | - | Enabled | - | Pressure-controlled venting is active | Model configuration |
|  | Maximum pressure | Pmax | 1.33×105 | Pa | Upper pressure limit for venting | Model parameter |
|  | Minimum pressure | Pmin | 1.20×105 | Pa | Lower pressure limit / vent shut-off pressure | Model parameter |
|  | Pressure smoothing | ΔPsmooth | 4.0×103 | Pa | Smoothing parameter controlling vent pressure transition | Model parameter |
|  | Vent path | - | VCS | - | Venting path through the VCS configuration | Model configuration |
|  | Flow model | - | Orifice | - | Orifice-based vent-flow model | Model configuration |
|  | Discharge coefficient | Cd | 0.62 | - | Orifice discharge coefficient | Model parameter |
|  | Vent area | Avent | 1.0×10-5 | m² | Orifice area | Model parameter |
|  | Back pressure | Pback | 1.01325×105 | Pa | Downstream/back pressure | Model parameter |
|  | Maximum solver step | Δtvent,max | 5.0 | s | Maximum time step during venting; solver guard | Solver parameter |
|  | Maximum vent events | Nevents,max | 5000 | - | Maximum number of permitted venting events | Solver parameter |
|  | Minimum dwell time | tdwell,min | 30 | s | Minimum time between successive venting events | Solver parameter |

# Appendix G

## G.1 Stochl tank insulation temperature profile validation

Figure 1. Steady state heat flux across the MLI at 294 K for the Stochl tank

# Appendix H

Figure 2. Study 1 integration points convergence study

## H.1 Parametric study 1: Insulation configuration comparison

Figure 3. MHTB and Stochl tank pressure history at 305 and 294 K respectively.

Figure 4. MHTB and Stochl tank liquid hydrogen fill level  history at 305 and 294 K respectively.

# Appendix I

## I.1 Parametric study 2: Ambient temperature sensitivity

Figure 5. Study 2 model integration points convergence study at 150 K.

Figure 6. Study 2 model integration points convergence study at 250 K.

Figure 7. Study 2 model integration points convergence study at 305 K.

Figure 8. Study 2 model integration points convergence study at 350 K.

Figure 9. MHTB tank foam steady state temperature from low to high temperatures

Figure 10. MHTB tank VCS steady state temperature at various ambient temperatures

Figure 11. BO reduction with reference to the Base insulation of the MHTB tank at different ambient temperature.

Figure 12. MHTB tank thermal history , (a) inner wall temperature , (b) foam temperature , and (c) VCS shield temperature at 2000 hours

Figure 13. MHTB tank inner wall heat flux history at 305K ambient temperature.

# Appendix J

## J.1 Parametric study 3: Foam thickness optimization

Figure 14. Model integration points convergence study at 305 K with varying foam thickness.

Figure 15. Model integration points convergence study at 305 K with varying foam thickness and VCS.

Figure 16. MHTB tank foam temperature with increasing foam thickness at 50 hr

Figure 17. MHTB tank foam temperature with increasing foam thickness

# Appendix K

## K.1 Parametric study 4: VCS flow fraction parametric

Figure 18. Model integration points convergence study at 300 K with varying VCS flow fraction and with 35 mm foam thickness.

Figure 19. Model integration points convergence study at 300 K with varying VCS flow fraction and without foam.

# Appendix L

## L.1 Parametric study 5: Mission performance and hold time

Figure 20. Deep space convergence study

Figure 21. LEO coast convergence study.

Figure 22. MHTB tank pressure history, (a) Ground hold( 305 K, 7 days), (c) LEO cost (250 K, 30 days), (c) Deep space ( 150 K, 180 days) , and (d) Lunar surface ( 220 K, 14 days )

# References

[1]	Leachman JW, Jacobsen RT, Penoncello SG, Lemmon EW. Fundamental equations of state for parahydrogen, normal hydrogen, and orthohydrogen. J Phys Chem Ref Data 2009;38:721–48. https://doi.org/10.1063/1.3160306.

[2]	Saturation Properties for Hydrogen n.d. https://webbook.nist.gov/cgi/fluid.cgi?PLow=0.10&PHigh=0.75&PInc=0.01&Digits=5&ID=C1333740&Action=Load&Type=SatT&TUnit=K&PUnit=MPa&DUnit=kg%2Fm3&HUnit=kJ%2Fkg&WUnit=m%2Fs&VisUnit=Pa*s&STUnit=N%2Fm&RefState=DEF (accessed September 21, 2026).

[3]	Saturation Properties for Hydrogen n.d. https://webbook.nist.gov/cgi/fluid.cgi?TLow=18&THigh=30&TInc=0.03&Digits=5&ID=C1333740&Action=Load&Type=SatP&TUnit=K&PUnit=MPa&DUnit=kg%2Fm3&HUnit=kJ%2Fkg&WUnit=m%2Fs&VisUnit=Pa*s&STUnit=N%2Fm&RefState=DEF#Auxiliary (accessed September 21, 2026).

[4]	Stochl RJ. BASIC PERFORMANCE OF A MULTILAYER INSULATION SYSTEM CONTAINING 20 T O 160 LAYERS. n.d.

[5]	Stochl RJ, Knoll RH. NASA Technical Memorandum 104476. n.d.

[6]	Martin JJ, Marshall LH. Large-Scale Liquid Hydrogen Testing of a Variable Density Multilayer Insulation With a Foam Substrate. n.d.

[7]	Hastings LJ, Hedayat A, Brown TM, Marshall GC. Analytical Modeling and Test Correlation of Variable Density Multilayer Insulation for Cryogenic Storage. 2004.

[8]	Hastings LJ, Flachbart RH, Martin JJ, Hedayat A, Fazah M, Lak T, et al. Spray Bar Zero-Gravity Vent System for On-Orbit Liquid Hydrogen Storage. 2003.